You are working in an existing robot-football simulation repository. The simulation environment, low-level controllers, and the following three skills are already implemented:

- reposition

- dribble

- shoot

There are two controlled quadruped robots on the same team. Each robot selects one skill and provides three continuous command parameters to that skill. The opponent is simplified as one or more static cylinders. The cylinders do not move or take actions.

Your task is to implement the complete pipeline for collecting data and training a skill-level world model that will later be used by model predictive control.

Do not implement MAPPO or the full MPC optimizer in this task. However, implement a clean, vectorized world-model rollout interface that can later be called by CEM, MPPI, or another MPC optimizer.

# 1. First inspect the existing repository

Before writing code, inspect the repository and identify:

- the environment class;

- the environment reset and step interfaces;

- whether the environment is vectorized;

- the high-level skill-selection interface;

- how the three continuous skill parameters are passed to each skill;

- the control frequency;

- the simulation frequency;

- how many low-level simulation steps correspond to one high-level action;

- the observation definitions;

- the global privileged state, if available;

- the reward computation;

- termination and truncation conditions;

- robot, ball, cylinder, and goal state tensors;

- the configuration system;

- the logging system;

- the existing checkpoint format;

- the available dependencies.

Do not invent environment APIs that do not exist. Adapt the implementation to the actual repository.

Do not modify the behavior of the existing environment, rewards, or low-level skills unless a small adapter is required. Keep environment-specific extraction logic isolated in a state adapter.

After inspecting the repository, provide a short implementation plan and then implement it.

# 2. World-model objective

Implement an ensemble world model that predicts the task-level transition over one high-level macro step:

[ f_{m}(s_t, a_t)  ( {t+1}, _t, _t, _t ), ]

where:

- (m) is an ensemble member;

- (s_t) is a compact global football state;

- (a_t) is the joint skill-command action of the two controlled robots;

- (_{t+1}) is the predicted change in the dynamic part of the state;

- (_t) is the predicted environment reward;

- (_t) is the predicted termination probability;

- (_t) contains optional event predictions.

The transition must correspond to one high-level action interval, not one physics step.

The main interface should conceptually be:

next_state, reward, done_probability, event_predictions, uncertainty = (
    world_model.predict_next(state, joint_action)
)

The model must also support batched multi-step imagined rollouts:

rollout_result = world_model.rollout(
    initial_states,
    candidate_action_sequences,
    deterministic=True,
)

This rollout interface must be suitable for future MPC use.

# 3. High-level action representation

Each robot has a hybrid action:

robot_action = {
    skill_id,
    skill_parameters[3]
}

The joint action contains the actions of both robots.

Use the following skill mapping unless the repository already defines one:

REPOSITION = 0
DRIBBLE = 1
SHOOT = 2

Represent the skill using either:

- a learned skill embedding; or

- a one-hot encoding.

Use a learned embedding by default.

The world-model action encoder should receive:

robot_0_skill_id
robot_0_skill_parameters[3]
robot_1_skill_id
robot_1_skill_parameters[3]

The continuous parameters must be normalized using their actual valid command bounds from the environment or skill configuration.

Do not assume that the three skills share identical command bounds. Store separate parameter bounds for every skill.

Provide utilities for:

- validating skill IDs;

- clipping skill parameters to valid ranges;

- normalizing skill parameters;

- denormalizing skill parameters;

- creating random valid joint actions;

- masking unused parameters when necessary.

If some parameters are irrelevant for a particular skill, include a parameter mask in the action representation.

# 4. High-level control interval

Define a configurable high-level macro-step duration.

A high-level action should be held for a configurable number of low-level control steps:

world_model:
  macro_action_steps: <infer sensible default from repository>

At every macro step:

- extract the current compact world-model state;

- sample or select a joint skill-command action;

- execute the selected skills for macro_action_steps;

- accumulate the environment reward during the macro step;

- extract the next compact state;

- store the resulting transition.

If an episode terminates during the macro step, stop execution immediately and store the actual number of executed low-level steps.

Store the elapsed macro-step duration in the dataset if it can vary.

# 5. Compact world-model state

Do not train the high-level world model directly on raw joint positions unless repository inspection shows that they are necessary.

The default state should describe task-level football dynamics.

Create a dedicated environment adapter such as:

class FootballWorldModelStateAdapter:
    def extract_state(self, env) -> dict:
        ...

    def encode_state(self, state_dict) -> torch.Tensor:
        ...

    def decode_dynamic_state(self, tensor) -> dict:
        ...

    def apply_predicted_delta(
        self,
        current_state,
        predicted_delta,
    ):
        ...

The exact field names must follow the existing environment.

The default dynamic state should contain, when available:

## Robot features for each controlled robot

- planar position (x,y);

- base height;

- yaw represented using sine and cosine;

- roll and pitch;

- planar linear velocity;

- vertical velocity;

- angular velocity;

- whether the robot is fallen;

- whether the robot is in contact with the ball;

- current skill ID;

- previous skill command;

- optional gait phase or skill phase if required for Markovian prediction.

## Ball features

- planar position;

- height;

- linear velocity;

- angular velocity, if available;

- ball possession indicator;

- identity of the possessing robot, if available;

- whether the ball is inside either goal;

- whether the ball is out of bounds.

## Static cylinder features

For every static cylinder, include:

- planar position;

- radius;

- optional height;

- validity mask.

Because cylinder ordering should not affect the prediction, use a permutation-invariant set encoder such as DeepSets:

cylinder_embedding = rho(
    sum(mask_j * phi(cylinder_feature_j))
)

If there is always exactly one cylinder, the same interface should still work.

The cylinders are static context. Do not predict their next positions.

## Field and goal context

Include or make available:

- field dimensions;

- controlled-team goal location;

- opponent goal location;

- goal width;

- field boundaries.

Fixed field geometry may be stored in configuration instead of repeated in every transition.

## Optional contact and event features

If available, include:

- robot-ball contact;

- robot-cylinder collision;

- teammate collision;

- ball-cylinder collision;

- successful shot;

- goal scored;

- own goal;

- ball possession change.

Document every selected state feature, its units, and its tensor index.

Create a machine-readable state schema saved with every dataset and checkpoint.

# 6. Coordinate representation

Use a consistent field coordinate system.

Prefer one of the following:

- a fixed global field coordinate system; or

- a canonical team coordinate system in which the controlled team always attacks along the positive (x)-axis.

Use the canonical team coordinate system if the environment supports side switching.

Normalize positions by field dimensions.

Represent all angles robustly:

- encode absolute yaw using sin(yaw) and cos(yaw);

- use wrapped angle differences where needed;

- never regress an unwrapped absolute angle directly.

Predict state changes rather than absolute next states for dynamic continuous quantities:

delta_robot_position
delta_robot_velocity
delta_robot_orientation
delta_ball_position
delta_ball_velocity

For bounded or binary features, use separate classification heads rather than treating everything as an unconstrained regression target.

# 7. Markov-state verification

The skill-level transition may depend on hidden low-level state.

To reduce partial observability, include the following when available:

- previous skill ID;

- previous skill parameters;

- time since the current skill was activated;

- gait phase;

- dribble phase;

- shooting phase;

- low-level controller phase;

- recent ball-contact indicator.

Add a configuration option to append a short history of task-level states and actions:

world_model:
  history_length: 1

Support history_length > 1, but default to one current state plus the previous high-level action.

Document whether the selected state appears approximately Markovian.

# 8. Dataset collection

Implement a data-collection script, for example:

python scripts/collect_world_model_data.py \
    --config configs/world_model.yaml \
    --output data/world_model/train

The collector must support vectorized environments when available.

Store transitions as complete episodes, preserving temporal order.

Each transition should contain at least:

episode_id
step_id
state
joint_action
reward
next_state
terminated
truncated
elapsed_low_level_steps
event_labels
behavior_source

Also store unnormalized structured states when practical for debugging.

Use a sharded dataset format appropriate for the repository. Prefer an existing project format. Otherwise use compressed NumPy shards, HDF5, or another lightweight format already supported by the repository.

Do not add a large dependency solely for dataset storage.

# 9. Initial-state randomization

Use the environment’s existing reset randomization.

Ensure the dataset covers:

- ball near each robot;

- ball near the cylinders;

- ball near field boundaries;

- ball near both goals;

- robots facing different directions;

- robots near and far from the ball;

- teammate separation;

- robots near cylinders;

- shooting configurations;

- failed shooting configurations;

- possession and non-possession states;

- collision-recovery states;

- fallen-robot states, if recoverable.

If the environment exposes reset-state controls, implement optional targeted reset distributions for rare but important states.

Do not replace existing domain randomization. Reuse it.

# 10. Behavior policy for data collection

Purely uniform random actions are insufficient because they will produce many useless trajectories.

Implement a configurable mixture of behavior sources.

Default mixture:

data_collection:
  behavior_mixture:
    random_valid: 0.50
    scripted: 0.30
    existing_policy: 0.20

If no existing high-level policy is available, redistribute its probability between random and scripted actions.

## Random valid behavior

Sample:

- valid skill IDs;

- valid parameters conditioned on the selected skill;

- skill duration or persistence, if configurable.

Avoid resampling a completely unrelated skill every single macro step. Add a configurable probability of repeating the previous skill:

data_collection:
  repeat_previous_skill_probability: 0.35

Sample commands using a mixture of:

- uniform sampling over valid bounds;

- Gaussian sampling around zero or nominal commands;

- boundary-focused sampling;

- goal-directed parameter sampling.

## Scripted behaviors

Implement simple scripted high-level behaviors only for collecting useful data.

Examples:

- move toward the ball;

- reposition behind the ball relative to the opponent goal;

- dribble toward the opponent goal;

- dribble around a static cylinder;

- shoot when the robot-ball-goal alignment is reasonable;

- intentionally shoot from poor alignment;

- one robot approaches the ball while the other moves to support;

- one robot dribbles while the second robot repositions;

- collision avoidance around a cylinder;

- recovery after losing ball possession.

The scripts should call the already-trained skills. Do not implement new low-level control.

## Existing-policy behavior

If an existing heuristic or learned high-level policy exists, provide an adapter and use it for part of the collection.

Record the source of every transition:

random_valid
scripted
existing_policy
targeted_rare_event

# 11. Rare-event balancing

Goals, successful shots, ball possession changes, and collisions may be rare.

Add optional targeted collection and sampling support for:

- successful shots;

- failed shots;

- ball-cylinder collisions;

- robot-cylinder collisions;

- teammate collisions;

- ball possession acquisition;

- ball possession loss;

- goals;

- out-of-bounds events.

Do not duplicate identical transitions.

Store event labels and implement weighted minibatch sampling or event-aware sampling during training.

The default training sampler should combine:

- ordinary uniformly sampled transitions;

- contiguous trajectory subsequences;

- rare-event-enriched transitions.

# 12. Dataset splitting

Split datasets by complete episode, never by individual transition.

Use configurable ratios such as:

dataset:
  train_fraction: 0.80
  validation_fraction: 0.10
  test_fraction: 0.10

Use deterministic seeded splitting.

Ensure that transitions from one episode cannot appear in multiple splits.

Save split manifests.

Implement tests that verify no episode leakage.

# 13. Data normalization

Compute normalization statistics from the training split only.

Normalize:

- continuous state inputs;

- continuous action parameters;

- continuous state-delta targets;

- reward targets, optionally.

Do not normalize:

- skill IDs;

- binary flags;

- categorical event labels;

- validity masks.

Use robust standardization with a minimum standard deviation clamp:

std = torch.clamp(std, min=1e-6)

Save normalization statistics with the checkpoint.

Implement:

normalizer.normalize_state(...)
normalizer.denormalize_state(...)
normalizer.normalize_action(...)
normalizer.normalize_delta_target(...)
normalizer.denormalize_delta_prediction(...)

# 14. Model architecture

Implement a probabilistic ensemble.

Default ensemble size:

model:
  ensemble_size: 5

Each member should be independently initialized and trained using a bootstrap sample or independently shuffled minibatches.

A reasonable default architecture is:

## State encoder

- MLP for robot and ball dynamic features;

- permutation-invariant encoder for cylinders;

- optional embedding for current and previous skill IDs;

- concatenate all embeddings into a global state embedding.

## Action encoder

For each robot:

- skill embedding;

- normalized three-dimensional parameter vector;

- optional parameter-validity mask.

Combine both robot action embeddings into a joint action embedding.

## Transition trunk

Use an MLP with residual blocks, for example:

input
→ Linear
→ SiLU
→ LayerNorm
→ residual MLP blocks
→ shared latent representation

Default hidden sizes:

model:
  hidden_dims: [512, 512, 512]
  activation: silu
  layer_norm: true
  dropout: 0.0

Avoid dropout by default because ensemble disagreement will be used for epistemic uncertainty.

## Output heads

Implement separate heads for:

- dynamic state-delta mean;

- dynamic state-delta log variance;

- reward mean;

- optional reward log variance;

- termination logit;

- truncation logit, if useful;

- event logits.

Clamp predicted log variance to a stable range, for example:

model:
  min_log_variance: -10.0
  max_log_variance: 2.0

Use learnable or smooth log-variance bounds if convenient.

# 15. Probabilistic prediction loss

Use Gaussian negative log likelihood for continuous dynamic state deltas.

For target (y), predicted mean (), and predicted log variance (^2):

[ _{} =  . ]

Use configurable feature-group weights so ball prediction can receive more importance than less relevant robot quantities.

Suggested groups:

loss:
  robot_position_weight: 1.0
  robot_velocity_weight: 0.5
  robot_orientation_weight: 0.5
  ball_position_weight: 2.0
  ball_velocity_weight: 2.0
  reward_weight: 1.0
  termination_weight: 1.0
  event_weight: 1.0

Use:

- Gaussian NLL or Huber loss for reward;

- binary cross entropy for termination;

- binary cross entropy or categorical cross entropy for event labels;

- optional L2 regularization through AdamW.

The total one-step loss should be:

[ L = L_{}

- rL{}

- dL{}

- eL{}. ]

Log each component independently.

# 16. Multi-step training loss

One-step accuracy alone is not enough for MPC.

Add optional multi-step rollout training using contiguous trajectory subsequences.

Configuration:

training:
  sequence_length: 5
  multi_step_loss_weight: 0.25
  multi_step_start_epoch: 10
  teacher_forcing_probability_start: 1.0
  teacher_forcing_probability_end: 0.0

During multi-step training:

- start from a real state;

- predict the next state using the dataset action;

- optionally feed either the real next state or the predicted next state into the following step;

- continue for the configured sequence length;

- compare predicted states and rewards with the actual trajectory.

Use a scheduled teacher-forcing probability.

Do not propagate a rollout beyond an actual terminal transition.

Use discounted multi-step loss weights:

[ w_h = ^{h-1} ]

with configurable (), such as 0.8.

Implement multi-step training so it can be disabled if it destabilizes training.

# 17. Ensemble bootstrapping and uncertainty

Each ensemble member should train on a different bootstrap resampling or independently shuffled version of the training data.

At inference, compute:

- ensemble mean prediction;

- aleatoric uncertainty from each member’s predicted variance;

- epistemic uncertainty from disagreement among member means.

For a predicted next-state feature:

epistemic_variance = variance(member_means, dim=ensemble)
aleatoric_variance = mean(member_variances, dim=ensemble)
total_variance = epistemic_variance + aleatoric_variance

Expose uncertainty through the rollout API.

Provide scalar uncertainty summaries for future MPC, including:

- mean state uncertainty;

- maximum state uncertainty;

- ball-state uncertainty;

- reward uncertainty.

Do not combine uncertainty with reward inside the world-model class. Return it separately so the future MPC objective can use a configurable penalty.

# 18. World-model rollout interface

Implement an MPC-ready class such as:

class WorldModelEnsemble(nn.Module):
    def predict_next(
        self,
        states,
        joint_actions,
        deterministic=True,
    ):
        ...

    def rollout(
        self,
        initial_states,
        action_sequences,
        deterministic=True,
        stop_on_done=True,
    ):
        ...

Support shapes similar to:

initial_states:
    [batch, state_dim]

action_sequences:
    [batch, num_candidates, horizon, action_dim]

Return:

predicted_states:
    [batch, num_candidates, horizon + 1, state_dim]

predicted_rewards:
    [batch, num_candidates, horizon]

predicted_done_probabilities:
    [batch, num_candidates, horizon]

state_uncertainty:
    [batch, num_candidates, horizon]

reward_uncertainty:
    [batch, num_candidates, horizon]

event_probabilities:
    [batch, num_candidates, horizon, num_events]

The rollout must:

- be vectorized;

- run on GPU;

- avoid Python loops over candidates;

- permit a short Python loop over horizon if necessary;

- keep static cylinder and field context unchanged;

- update dynamic state features using predicted deltas;

- correctly renormalize sine and cosine orientation representations;

- stop or mask rewards after predicted termination;

- support deterministic ensemble-mean rollout;

- optionally sample stochastic predictions.

Add a method that computes a discounted predicted return without embedding any task-specific MPC optimizer:

evaluate_action_sequences(
    initial_states,
    action_sequences,
    gamma,
    uncertainty_penalty=0.0,
)

# 19. Training script

Implement a training entry point such as:

python scripts/train_world_model.py \
    --config configs/world_model.yaml

Use the repository’s configuration system if one exists.

Otherwise provide a YAML configuration.

Suggested defaults:

seed: 42

training:
  device: cuda
  batch_size: 1024
  sequence_batch_size: 256
  learning_rate: 0.0003
  weight_decay: 0.00001
  max_epochs: 200
  gradient_clip_norm: 10.0
  early_stopping_patience: 20
  mixed_precision: true
  num_workers: 4
  sequence_length: 5
  multi_step_loss_weight: 0.25
  multi_step_start_epoch: 10

model:
  ensemble_size: 5
  hidden_dims: [512, 512, 512]
  skill_embedding_dim: 16
  cylinder_embedding_dim: 64
  activation: silu
  layer_norm: true
  min_log_variance: -10.0
  max_log_variance: 2.0

Use AdamW.

Use validation loss for early stopping.

Save:

- latest checkpoint;

- best validation checkpoint;

- final checkpoint;

- individual ensemble-member checkpoints if useful.

A checkpoint must contain:

- model weights;

- optimizer state;

- scheduler state;

- normalizer statistics;

- state schema;

- action schema;

- skill parameter bounds;

- cylinder schema;

- training configuration;

- epoch;

- validation metrics;

- random seed;

- repository commit hash when available.

Support resuming training.

# 20. Evaluation

Implement:

python scripts/evaluate_world_model.py \
    --checkpoint checkpoints/world_model/best.pt \
    --dataset data/world_model/test

Report at least:

## One-step metrics

- normalized and physical-unit RMSE;

- MAE;

- negative log likelihood;

- robot position RMSE;

- robot velocity RMSE;

- ball position RMSE;

- ball velocity RMSE;

- reward RMSE;

- termination precision, recall, and F1;

- event precision, recall, and F1.

## Multi-step rollout metrics

Evaluate horizons:

1, 3, 5, 10, 20

Report:

- robot-position error versus horizon;

- ball-position error versus horizon;

- ball-velocity error versus horizon;

- cumulative reward error versus horizon;

- termination calibration versus horizon.

Use real contiguous test trajectories and their recorded action sequences.

## Uncertainty calibration

Measure whether ensemble disagreement correlates with actual prediction error.

Report:

- correlation between uncertainty and squared error;

- average error in uncertainty quantiles;

- error on in-distribution samples;

- error on deliberately unusual states, when available.

## Visualizations

Save plots for:

- training and validation losses;

- one-step prediction error by feature group;

- multi-step error versus horizon;

- predicted versus actual ball trajectories;

- predicted versus actual robot trajectories;

- reward prediction;

- uncertainty versus actual error;

- reliability plot for termination probabilities.

Use repository logging if available. Otherwise save plots and metrics to an output directory.

# 21. Environment-level validation

Implement an optional validation script that:

- resets the real simulator;

- samples a valid high-level action sequence;

- rolls the sequence through the learned world model;

- executes the same sequence open-loop in the simulator;

- compares predicted and actual trajectories.

Example command:

python scripts/validate_world_model_in_env.py \
    --checkpoint checkpoints/world_model/best.pt \
    --num-trajectories 100 \
    --horizon 10

This is for evaluation only. Do not call this open-loop execution strategy MPC.

# 22. Data-quality diagnostics

Before training, generate a dataset report containing:

- number of episodes;

- number of transitions;

- episode-length distribution;

- skill-frequency distribution for both robots;

- command-parameter distributions by skill;

- state-feature ranges;

- reward distribution;

- termination causes;

- event counts;

- number of goals;

- number of shots;

- successful-shot rate;

- cylinder-collision counts;

- possession-change counts;

- behavior-source distribution;

- train, validation, and test sizes.

Warn about:

- constant features;

- NaN or infinite values;

- impossible skill IDs;

- actions outside bounds;

- state discontinuities;

- duplicate transitions;

- episode leakage;

- severe skill imbalance;

- absent rare events.

# 23. Tests

Add unit and integration tests using the repository’s test framework.

At minimum test:

- state extraction returns finite tensors;

- state encoding and decoding preserve required fields;

- angle wrapping is correct;

- yaw sine and cosine remain normalized after prediction;

- static-cylinder context remains unchanged during rollout;

- action normalization and denormalization are inverses;

- skill-dependent action bounds are respected;

- invalid skill IDs raise clear errors;

- dataset episode splitting has no leakage;

- ensemble output shapes are correct;

- uncertainty is nonnegative;

- one-step rollout output shapes are correct;

- multi-step rollout output shapes are correct;

- terminated trajectories are correctly masked;

- a tiny dataset can be overfit;

- one optimizer step decreases or preserves a deterministic toy loss;

- checkpoint save and load preserve predictions;

- CPU inference works;

- CUDA inference works when CUDA is available;

- vectorized environment data collection works, if the environment is vectorized.

Create a small smoke test that:

- collects a small dataset;

- trains for a few epochs;

- saves a checkpoint;

- reloads the checkpoint;

- performs a five-step imagined rollout.

# 24. Suggested project structure

Adapt to the repository, but keep responsibilities separated. A possible structure is:

world_model/
    __init__.py
    state_adapter.py
    action_adapter.py
    schema.py
    dataset.py
    normalizer.py
    behavior_policies.py
    model.py
    ensemble.py
    losses.py
    trainer.py
    rollout.py
    metrics.py
    visualization.py

scripts/
    collect_world_model_data.py
    inspect_world_model_dataset.py
    train_world_model.py
    evaluate_world_model.py
    validate_world_model_in_env.py

configs/
    world_model.yaml

tests/
    test_state_adapter.py
    test_action_adapter.py
    test_world_model_dataset.py
    test_world_model.py
    test_world_model_rollout.py

Do not force this structure if the repository already has a strong convention.

# 25. Implementation quality requirements

Use:

- Python type hints;

- docstrings for public classes and functions;

- dataclasses or typed configuration objects where appropriate;

- deterministic seed handling;

- clear tensor-shape comments;

- explicit device handling;

- meaningful error messages;

- no hard-coded state indices outside the state schema;

- no hard-coded action bounds;

- no silent clipping of invalid data during training;

- no dependence on GUI rendering;

- no environment behavior changes.

Keep model code independent of the simulation backend.

All simulation-specific logic must remain in adapters and collection scripts.

# 26. Documentation

Create documentation explaining:

- what one world-model transition represents;

- the state definition;

- the action definition;

- how static cylinders are encoded;

- how data are collected;

- how train-validation-test splitting works;

- how normalization works;

- the model architecture;

- the loss functions;

- ensemble uncertainty;

- how to train;

- how to evaluate;

- how to load a checkpoint;

- how future MPC code should call rollout;

- known limitations.

Include runnable commands using the actual repository paths.

# 27. Important design constraints

Follow these constraints:

- The opponent cylinders are static and have no action.

- Do not predict cylinder motion.

- Do not implement an opponent policy.

- The two controlled robots are jointly modeled.

- Use actual simulator transitions as training targets.

- Do not train from recursively imagined transitions as if they were ground truth.

- Keep train, validation, and test episodes separate.

- Use the learned model only at the high-level skill timescale.

- Do not replace the existing low-level policies.

- Do not implement MAPPO in this task.

- Do not implement a complete MPC optimizer in this task.

- Do implement an MPC-ready vectorized rollout interface.

- Do not assume a specific simulator library before inspecting the repository.

- Do not silently invent observation fields that the environment does not expose.

# 28. Recommended staged implementation

Implement in this order:

## Stage A: Repository integration

- inspect environment APIs;

- implement state and action adapters;

- implement schemas;

- write adapter tests.

## Stage B: Data collection

- implement random valid behavior;

- implement scripted behavior;

- collect transitions;

- store episodes;

- implement dataset diagnostics.

## Stage C: Deterministic baseline

Before the probabilistic ensemble, implement a simple deterministic one-step MLP baseline.

Verify that:

- the data pipeline is correct;

- a tiny dataset can be overfit;

- prediction targets and normalization are correct.

Keep this baseline available through configuration.

## Stage D: Probabilistic ensemble

- implement probabilistic heads;

- implement ensemble members;

- implement bootstrap training;

- implement uncertainty estimation.

## Stage E: Multi-step training and evaluation

- implement contiguous sequence loading;

- implement optional multi-step loss;

- implement horizon-based evaluation.

## Stage F: MPC-ready rollout

- implement vectorized candidate-sequence rollout;

- implement discounted-return evaluation;

- implement uncertainty output;

- add rollout tests and benchmarks.

# 29. Acceptance criteria

The work is complete when all of the following are satisfied:

- A command can collect a dataset from the existing simulation.

- The dataset includes complete episode structure.

- The dataset contains valid joint skill-command actions.

- The dataset inspection command reports meaningful statistics.

- A deterministic baseline can overfit a tiny dataset.

- The probabilistic ensemble trains successfully.

- Training and validation losses are logged.

- One-step test metrics are produced.

- Multi-step horizon metrics are produced.

- Ensemble uncertainty is returned.

- A saved checkpoint can be reloaded exactly.

- The rollout API supports batched candidate action sequences.

- Static cylinders remain unchanged during imagined rollout.

- The implementation has unit and smoke tests.

- The documentation contains runnable repository-specific commands.

At the end, provide:

- a summary of the repository APIs you found;

- a list of created and modified files;

- exact commands for data collection;

- exact commands for training;

- exact commands for evaluation;

- exact commands for the smoke test;

- any assumptions that could not be verified from the repository;

- remaining limitations or recommended follow-up work.